# Session 3: Build UWP apps and drivers and making Imaging with them

# Build UWP apps , debugging it and create an image with it

PURPOSE: Understand how easy it is to develop app and create custom image

Reference:  Adding Apps – Adding Apps

- Enable Developer mode, Settings -> Update & Security -> Developer mode

- Launch Visual Studio 2017

- Go to File -> New -> Projects -> Visual C# -> Windows Universal -> Blank App, create new project name

- Create new project “Sample” and select target version “1809” and keep minimum version to default.

[Note] if you change the minimum to 1809 then you will not see appx but msix, so suggest to keep default for the lab.

- Select target platform to ARM, optionally you can customize resolution information by double clicking the MainPage.xaml in Solution Explorer.

- Enable toolbox  with Ctrl + Alt + X or goes to View -> Toolbox , enable All XAML Controls

- Select TextBox control and Put any text in UX page. You can customize text size in “Properties” box

- Build solution

- Confirm our PC and RPi3 device is under the same subnet

- Configure Remote machine to deploy and debug, Device -> Remote Machine, or Solution Explorer -> Property -> Debug ->   Remote machine -> Find, select your device name or IP address.

[Note] If you don’t see the list then you can just put the IP address directly

- Press F5 or click “Remote Machine” for debugging

- To stop debugging, press Shift + F5

- In Visual Studio, to create your application as an Appx package. This is done by clicking Project > Store > Create App Packages > I want to Create Packages for Sideloading > Next

- Select Generate app bundle: Never, and select ARM only as we only target ARM device

- Click Create

- After completing package completion, you can access the package

- Open IoTCorePShell.cmd from your Workspace.  It should prompt you to run as an administrator.

- Create the package for your Appx by using Add-IoTAppxPackage. In our example, the command is as follows:

Add-IoTAppxPackage C:\Users\...\Sample_1.0.1.0_ARM_Debug_Test\Sample_1.0.1.0_ARM_Debug.appx fga APPX.SAMPLE

[Note] The fga parameter indicates the Appx file is a foreground application. If you specify your package as a background application (with the bga parameter) and have no other foreground applications in the image, the system will get stuck when booting up (displays a spinner indefinitely).

[Note]Be aware that if your Appx has dependencies you will need the Dependencies subdirectory to be present in the same location as your Appx when you run this command. If not then this will result in errors when you build your FFU image.

- From IoT Core Powershell Environment, you can now build the package into a .CAB file using New-IoTCabPackage. This will build the package into a .CAB file under [Your Workspace] \Build\<arch>\pkgs

Ex) New-IoTCabPackage APPX.SAMPLE

- Add the Feature ID for your app package using Add-IoTProductFeature. This adds a FeatureID called APPX_SAMPLE  to the specified product's Test OEMInput XML file and include the package to the image.

Ex) Add-IoTProductFeature ProductA Test APPX_SAMPLE -OEM

Test OEMInput XML file : C:\MyWorkspace\Source-arm\<product name>\TestOEMInput.xml file

- Build the FFU image again. You should only have to run the New-IoTFFUImage command:

Ex) New-IoTFFUImage ProductA Test

[Tip]If you see New-IoTFIPPackage fail error as below then try New-IoTCabPackage All

Then, run New-IoTCabPackage All again and try build image again.

# Build KERNEL DRIVERS and Create an image with it

PURPOSE : Understand how the driver is made and integrated to the image

Reference, Adding Apps – Adding Drivers

- As Community version is not supporting driver compile, so if you use Community version then skip to build KMDF drivers and use complied one under HOL folder

- Using Visual Studio, load gpiokmdfdemo project. Confirm Project -> Property

- Compile the gpiokmdfdemo project, located at [HOL folder]\Programs \Gpiokmdfdemo

[Tip] If you see code signing error, then please specify test cert as following

“Windows OEM Test Cert 2017 (TEST ONLY)”

- Copy the compiled driver files into a test folder (for example, c:\temp\gpiokmdfdemo) from your [HOL folder] \Programs\Gpiokmdfdemo\ARM\Debug

gpiokmdfdemo.sys and gpiokmdfdemo.inf

- Run IoT Core Powershell Environment from your Worksapce.

- Create a driver package using Add-IoTDriverPackage:

This creates a new folder at C:\MyWorkspace\Source-<arch>\Packages\Drivers.TestDriver. This also adds a FeatureID called DRIVERS_TESTDRIVER to the C:\MyWorkspace\Source-<arch>\Packages\OEMFM.xml file

Ex) Add-IoTDriverPackage C:\temp\gpiokmdfdemo\gpiokmdfdemo.inf Drivers.TestDriver

- Build the driver package using New-IoTCabPackage

Ex) New-IoTCabPackage Drivers.TestDriver

- Enable the feature to add the driver package to the image. Update the product test configuration file using Add-IoTProductFeature:

Ex) Add-IoTProductFeature ProductA Test DRIVERS_TESTDRIVER -OEM

- Build the FFU image again, as specified in Creating a Basic IoT Core Image. You should only have to run the New-IoTFFUImage command:

Ex) New-IoTFFUImage ProductA Test

[Tip]if you see following Get-ChildItem error then run it again after reboot.